I am using RPLidar a3, cmp10A yahboom for 2D mapping. and yolo v8 model and a intel realsense d435i for object detection and axis finding. I have a package for Cartographer SLAM with lidar and imu ros2 packages inside. then I have a yolovnv package for running yolo. The pt files are at a different place. I have done gui for the yolo object detection and updating on the terminal as it is detecting a object according to the format. Now I have to finalize everything. I want all the package in a same folder. Then when I start a 2D map for   I have to detect object, update it on the map then save the map in GeoTiff and save the final map.

Here are the rules:

Mapping and Object Detection Format Specifications 2D Map Format 2D maps have to be submitted in the specified GeoTIFF format. An open-source implementation for ROS is available at http://wiki.ros.org/hector_geotiff.

In the following, all elements of the map are described: • Fileformat: GeoTIFF Version: 2026B • FILENAME: DARK BLUE (RGB: 0, 44, 207) TEXT For example, “RoboCup2025-TeamName-Mission1.tiff” displayed in the upper left corner to identify the map, make it sort properly in a directory, and findable on a computer. • MAP SCALE: DARK BLUE (RGB: 0, 50, 140) TEXT AND EXACTLY 1 METER LONG LINE Display this in the upper right corner to indicate the scale of the map. • MAP ORIENTATION: DARK BLUE (RGB: 0, 50, 140) TEXT (“X” AND “Y”) AND ABOUT 50 cm LONG ARROWS Display this next to the map scale. It gives the orientation for the victim location in the victim file. Must be a right handed coordinate system: X points upwards, Y to the left. • UNEXPLORED AREA GRID: LIGHT/DARK GREY (RGB: 226, 226, 227/RGB: 237, 237, 238) CHECKERBOARD WITH 100CM SQUARES This solid checkerboard pattern should show the unexplored area and provide scale on all sides of the mapped area. It should also print in black and white without ambiguity with other areas potentially turned grey in the process. • EXPLORED AREA GRID: BLACK (RGB: 190,190,191) GRID WITH 50CM GRID AND ABOUT 1 CM THICK LINES (use a one pixel line in the map) This grid should only appear in the explored area, behind any walls, victim locations, or other information. The grid should be aligned with the checkerboard pattern of the unexplored area, but twice as fine to allow visual inspection of wall alignments. • INITIAL ROBOT POSITION: GREEN (RGB: 0, 240, 0) ARROW This should mark the initial position of the robot and always be pointed toward the top of the map. • WALLS AND OBSTACLES: DARK BLUE (RGB: 0, 40, 120) FEATURES This should indicate the walls and other obstacles in the environment. The color should make the walls stand out from everything else. • SEARCHED AREA: WHITE CONFIDENCE GRADIENT (RGB: 128, 128, 128 to RGB: 255, 255, 255) This should be based on the confidence that the area is really free. It should produce a clean white when seen as free by all measurements and nearly untouched when undecided, that is, nearly equally seen as occupied as free, to produce a dither effect. • APRILTAG LOCATION: SOLID YELLOW (RGB: 255, 200, 0) CIRCLE WITH ABOUT 35CM DIAM CONTAINING WHITE (RGB) TEXT “#” REPRESENTING THE LABEL ASSOCIATED WITH THE APRILTAG • HAZMAT LOCATION: SOLID ORANGE (RGB: 255, 100, 30) DIAMOND WITH ABOUT 30CM SIDES CONTAINING WHITE (RGB) TEXT “@@” REPRESENTING THE FIRST TWO LETTERS OF THE HAZMAT LABEL • REAL OBJECT LOCATION: SOLID RED (RGB: 240, 10, 10) DIAMOND WITH ABOUT 30CM SIDES CONTAINING WHITE (RGB) TEXT “@@” REPRESENTING THE FIRST TWO LETTERS OF THE IDENTIFIED OBJECT • ROBOT PATH: MAGENTA (RGB: 120, 0, 140) LINE ABOUT 2CM THICK This should show the robot path. NOTE: If a team wishes to deviate from this 2D format in order to make improvements on the understandability and usefulness of their generated 2D map, they will not be penalized for doing so. The submitted map should contain all of the same information (scale, detections, physical barriers, robot path, etc.), but stylistic choices may be made to make it more presentable. The important capability to demonstrate is producing a clean, useful 2D map for field operations.

Identified Objects CSV Format In addition to indicating identified objects in the map, they have to be submitted in a CSV list. In the following, the structure of the file is given: Filename: RoboCup[Year]-[Teamname]-[Mission]-[Start Time]-pois.csv ([Start Time] should be formatted as HH-MM-SS) Format for the file header: Replace the placeholders “[...]” with your data. Please leave the quotes in the file, but not the brackets. The file body contains one line for each found object in the following format: • [detection]: Unique integer counter for each object that is also printed on the geotiff map • [time]: Time stamp when the object was found • [type]: Type of the object: [ ar_code, hazmat_sign, real_object, heat_sig ] • [name]: Unique ID for detection (i.e. ar_code number, hazmat label, object name) • [x], [y], [z]: Coordinates of the object in meters • [robot]: Name of the robot that found the object • [mode]: Mode of the robot when it found the object: A for autonomous, T for teleoperated Strings which include spaces should be enclosed with double quotes. Here is an example of a file which might be named “RoboCup2025-ReskoKoblenz-Prelim1-pois.csv”: IMPORTANT: There will only be one instance of each object included in the mapping area at any time. Teams must only include one detection per unique object. If duplicate objects are contained in the submitted CSV, only the first instance of that object will be used for scoring. A list of object labels will be provided to teams. Mislabelled objects and submissions not following the specified format will not be scored. [detection],[time],[type],[name],[x],[y],[z],[robot],[mode] "pois" "1.3" "Resko Koblenz" "Germany" "2018-06-23" "14:37:03" "Prelim1" detection,time,type,name,x,y,z,robot,mode 1,14:28:01,”ar_code”,”2”,-8.2992,-2.2904,0.49,"Robbie 1",A 2,14:28:02,”ar_code”,”34”,-8.2993,-2.2902,0.4563,"Robbie 2",T 3,14:28:05,”hazmat_sign”,”poison”,-5.7452,-7.0849,0.2130,"Robbie 1",A 4,14:30:14,”real_object”,”gloves”,-6.0845,-0.1251,0.3561,"Robbie 1",A 5,14:32:56,”heat_sig”,”0”,-9.2817,0.04968,0.7561,"Robbie 2",T "pois" "1.3" "[Your team name]" "[Your country]" "[Start Date]" "[Start Time]" "[Mission #]" detection,time,type,name,x,y,z,robot,mode

3D Point Cloud Map Teams will submit a 3D pointcloud representation of the mapped area to be scored with respect to a generated ground truth map. The submitted map should adhere to the following specification: Filename: RoboCup[Year]-[Teamname]-[Mission]-[Start Time]-map.ply ([Start Time] should be formatted as HH-MM-SS) Your point cloud should be formatted as an ASCII file. The header of your file should adhere to the PLY file format. At a minimum, your file should include float type fields for x, y, and z. Teams may include additional fields such as:-RGB Color Information-Heat Information-Point Normal Information-Confidence Scalar Maps which contain sensible color and/or heat data will be awarded a bonus point multiplier. IMPORTANT: The scale of your map coordinates are expected to be in meters. (i.e. A distance of 1 unit in the submitted point cloud should correspond to 1 meter in the real world). Improper scaling will severely penalize your score. The origin location (0,0,0) of the 3D Cloud should be set to the starting position for the robot which will be marked on the floor in the mapping area. More specifically, the origin should be set to be the center of the front of the robot, at floor height. Rotationally, the starting position will point the robot in the +Y direction, and the vertical axis will be Z. More information on PLY format can be found here. The format for the PLY header that should be included is shown below. NOTE: You are not required to include all fields if you are not collecting those kinds of data ply format ascii 1.0 comment {Team Name} comment {Start time} comment {Mission #} element vertex {Number of Vertices} property float x property float y property float z property uchar red property uchar green property uchar blue property float nx property float ny property float nz property float temp property float confidence end_header {DATA}

Guide me.

========================================================================

What you need now is:

- A clean architecture

- A single ROS2 workspace

- A central mission manager node

- GeoTIFF export

- Object fusion into map coordinates

- CSV logger

- Final packaging/export pipeline

This is no longer “just ROS packages.”
 This becomes a competition-grade autonomous mapping system.

# 1️⃣ YOUR FINAL TARGET ARCHITECTURE

You currently have:

| Component | Status |
| --- | --- |
| RPLidar A3 | ✅ |
| CMP10A IMU | ✅ |
| Cartographer SLAM | ✅ |
| D435i | ✅ |
| YOLOv8 | ✅ |
| GUI detections | ✅ |
| Terminal updates | ✅ |

Now combine them into:

Sensors

├── RPLidar A3

├── CMP10A IMU

└── Intel RealSense D435i

│

▼

ROS2 Drivers

├── rplidar_ros2

├── cmp10a_driver

└── realsense2_camera

│

▼

Cartographer SLAM

│

▼

/map  + TF + robot pose

│

├──────────────┐

▼              ▼

GeoTIFF Export      Object Localization Node

│

YOLOv8 + Depth

│

3D Object Position

│

CSV Logger

│

Map Annotation

# 2️⃣ SINGLE CLEAN WORKSPACE (VERY IMPORTANT)

Right now your packages are probably scattered.

You should create:

rrl_ws/

├── src/

│    ├── rplidar_ros2/

│    ├── cmp10a_ros2/

│    ├── realsense_ros/

│    ├── cartographer_ros/

│    ├── yolov8_detector/

│    ├── object_localizer/

│    ├── geotiff_mapper/

│    ├── csv_logger/

│    ├── mission_manager/

│    └── bringup/

│

├── maps/

├── logs/

├── models/

│    └── best.pt

├── exports/

└── install/

# 3️⃣ MOVE YOLO MODEL INTO WORKSPACE

DO NOT keep .pt elsewhere.

Put it here:

rrl_ws/models/best.pt

Then in your node:

model_path = os.path.join(

get_package_share_directory('yolov8_detector'),

'../../models/best.pt'

)

OR cleaner:

yolov8_detector/

├── models/

│    └── best.pt

4️⃣ CRITICAL PART → OBJECT LOCALIZATION

THIS is the hardest part.

YOLO only gives:

(u,v) image coordinates

RRL requires:

(x,y,z) in world coordinates

So pipeline becomes:

Step A — Detect Object

YOLO:

bbox = [x1,y1,x2,y2]

label = "gloves"

Step B — Get Depth

From D435i depth frame:

depth = depth_image[center_y, center_x]

Step C — Convert Pixel → Camera 3D

Use RealSense intrinsics:

X = (u - cx) * depth / fx

Y = (v - cy) * depth / fy

Z = depth

## Step D — Transform to MAP frame

Use TF2:

camera_link  → base_link → map

Then:

tf_buffer.lookup_transform(

"map",

"camera_link",

rclpy.time.Time()

)

Now you get:

map_x

map_y

map_z

THIS is what goes into:

- GeoTIFF markers

- CSV

- Final scoring

# 5️⃣ CREATE A DEDICATED OBJECT DATABASE

You MUST avoid duplicate detections.

Create:

detected_objects = {}

Example:

{

"gloves": {

"x": -2.3,

"y": 1.2,

"z": 0.5,

"time": "14:30:22"

}

}

Before adding new object:

if distance(existing,new) < threshold:

ignore

RRL explicitly penalizes duplicates.

# 6️⃣ GEO TIFF GENERATION (VERY IMPORTANT)

You already found the correct requirement.

Use:

hector_geotiff

ROS package:

hector_geotiff_plugins

## Install

sudo apt install ros-humble-hector-geotiff

## Export map

ros2 run hector_geotiff geotiff_saver

OR launch node.

# 7️⃣ BUT RRL FORMAT IS CUSTOM

This is VERY important.

Default hector_geotiff output is NOT enough.

You must overlay:

- scale

- robot path

- object icons

- labels

- coordinate arrows

- grids

- checkerboard

So you need:

# 8️⃣ CREATE A CUSTOM MAP RENDERER NODE

This node:

Subscribes:

- /map

- /tf

- /detected_objects

- /path

Outputs:

- Final RRL GeoTIFF

## Recommended pipeline

OccupancyGrid

↓

Convert to OpenCV image

↓

Draw overlays

↓

Save TIFF

# 9️⃣ YOU SHOULD USE OpenCV FOR DRAWING

Example:

cv2.line()

cv2.putText()

cv2.circle()

cv2.polylines()

# 🔟 THINGS YOU MUST DRAW

## Required by RRL

| Item | Draw |
| --- | --- |
| Walls | Occupancy grid |
| Robot path | Magenta polyline |
| Objects | Red diamonds |
| Hazmat | Orange diamonds |
| AprilTags | Yellow circles |
| Origin | Green arrow |
| Scale | Top-right |
| X/Y axis | Top-right |
| Grid | 50 cm |
| Checkerboard | unexplored |

# 1️⃣1️⃣ ROBOT PATH TRACKING

Subscribe to:

/tf

Store robot poses periodically:

path_points.append((x,y))

Draw at end:

cv2.polylines()

# 1️⃣2️⃣ CSV LOGGER NODE

You NEED a dedicated CSV node.

Output:

RoboCup2026-Team-Mission-HH-MM-SS-pois.csv

## CSV Entry Example

1,14:28:01,"real_object","gloves",-2.3,1.2,0.4,"Robot1",A

# 1️⃣3️⃣ FINAL FILES YOU MUST PRODUCE

At mission end:

exports/

├── RoboCup2026-Team-Mission1.tiff

├── RoboCup2026-Team-Mission1-pois.csv

├── RoboCup2026-Team-Mission1-map.ply

# 1️⃣4️⃣ 3D POINT CLOUD EXPORT

You can generate from D435i:

Use:

/open3d

OR RealSense pointcloud.

Save:

o3d.io.write_point_cloud("map.ply", pcd)

Must be:

ASCII PLY

# 1️⃣5️⃣ MOST IMPORTANT NODE → MISSION MANAGER

Create:

mission_manager

This node:

- starts mission timer

- launches SLAM

- launches YOLO

- records detections

- triggers GeoTIFF save

- triggers CSV export

- triggers PLY export

This becomes your competition control center.

# 1️⃣6️⃣ RECOMMENDED FINAL ROS2 PACKAGES

You should end with:

| Package | Purpose |
| --- | --- |

| bringup | Launch all |
| --- | --- |

| cartographer_config | SLAM |
| --- | --- |

| yolov8_detector | Detection |
| --- | --- |

| object_localizer | 3D coordinates |
| --- | --- |

| detection_database | Duplicate filtering |
| --- | --- |

| geotiff_renderer | Final TIFF |
| --- | --- |

| csv_logger | CSV |
| --- | --- |

| pointcloud_exporter | PLY |
| --- | --- |

| mission_manager | Orchestration |
| --- | --- |

# 1️⃣7️⃣ YOUR MOST CRITICAL TECHNICAL PROBLEM

This one:

YOLO pixel → world coordinate

THIS determines whether:

- detections align on map

- CSV coordinates are valid

- scoring works

Spend most effort here.

# 1️⃣8️⃣ MY RECOMMENDED STACK FOR YOU

Given your hardware:

| Task | Best Option |
| --- | --- |
| SLAM | Cartographer |
| Mapping | OccupancyGrid |
| GeoTIFF | Hector + OpenCV |
| Object Detection | YOLOv8 |
| Object Localization | TF2 + depth |
| Point Cloud | Open3D |
| GUI | RViz2 |
| Coordination | ROS2 lifecycle nodes |

# 1️⃣9️⃣ WHAT YOU SHOULD DO NEXT (IN ORDER)

## PHASE 1

✅ Clean workspace
✅ Move packages together
✅ Unified launch file

## PHASE 2

✅ Reliable SLAM
✅ Stable TF tree
✅ Stable map saving

## PHASE 3

✅ Object localization in MAP frame
✅ Duplicate filtering

## PHASE 4

✅ GeoTIFF renderer
✅ CSV logger

## PHASE 5

✅ Final mission export pipeline

# 2️⃣0️⃣ YOUR FINAL DELIVERABLE

At competition:

ros2 launch bringup mission.launch.py

Robot should:

- map

- detect

- localize

- annotate

- export everything automatically

THAT is a proper RRL system.